Networking Lab 13

Subnetting Hands-On – Network Range and Subnet Mask Analysis

Objective

The objective of this lab was to understand the fundamentals of IPv4 subnetting by calculating network ranges, network addresses, broadcast addresses, usable IP addresses, subnet masks, and the total number of addresses for different CIDR prefixes.

The lab also involved verifying the subnet configuration of the DC-Server and Win11-Client using ipconfig /all and practicing subnetting calculations using /24 and /8 networks.

Lab Environment

| Component | Details |
| --- | --- |
| Domain Controller | DC-Server |
| Client Machine | Win11-Client |
| Domain | mydomain.com |
| DC IPv4 Address | 172.16.0.1 |
| Client IPv4 Address | 172.16.0.50 or an address within the range |
| Subnet Mask | 255.255.255.0 |
| CIDR Prefix | /24 |
| Command Used | ipconfig /all |

Part 1: Calculate Network Range for 172.16.0.0/24

Step 1: Determine the Network Information

The network used for the lab is:

172.16.0.0/24

The /24 CIDR prefix means that the first 24 bits are used for the network portion, leaving 8 bits for hosts.

Subnetting Calculation

CIDR:        /24

Subnet Mask: 255.255.255.0

Host Bits:   8

Total IPs:   2⁸ = 256

Usable IPs:  256 - 2 = 254

Two addresses are reserved:

- Network address

- Broadcast address

Network Details

| Parameter | Value |
| --- | --- |
| IP Range | 172.16.0.0 – 172.16.0.255 |
| Network Address | 172.16.0.0 |
| Broadcast Address | 172.16.0.255 |
| Usable IPs | 172.16.0.1 – 172.16.0.254 |
| Subnet Mask | 255.255.255.0 |
| Total IPs | 256 |
| Usable IPs | 254 |

Observation

The /24 network provides 256 total addresses, of which 254 can be assigned to hosts.

Part 2: Comparing /24 with /16

Step 2: What If We Used 172.16.0.0/16?

The same private IP range can be represented using a /16 prefix:

172.16.0.0/16

The subnet mask becomes:

255.255.0.0

There are now 16 host bits.

Calculation

Host Bits = 32 - 16

= 16

Total IPs = 2¹⁶

= 65,536

Usable IPs = 65,536 - 2

= 65,534

Network Details

| Parameter | Value |
| --- | --- |
| IP Range | 172.16.0.0 – 172.16.255.255 |
| Network Address | 172.16.0.0 |
| Broadcast Address | 172.16.255.255 |
| Usable IPs | 172.16.0.1 – 172.16.255.254 |
| Subnet Mask | 255.255.0.0 |
| Total IPs | 65,536 |
| Usable IPs | 65,534 |

Observation

Changing from /24 to /16 significantly increases the number of available host addresses.

Part 3: Verify Subnet Mask on DC-Server

Step 3: Check Network Configuration

Procedure

On DC-Server:

- Open Command Prompt.

- Execute:

ipconfig /all

- Locate the network adapter information.

- Verify the IPv4 address and subnet mask.

Expected Configuration

IPv4 Address:  172.16.0.1

Subnet Mask:   255.255.255.0

Observation

The subnet mask 255.255.255.0 corresponds to:

/24

Result

The Domain Controller is configured on the 172.16.0.0/24 network.

Part 4: Verify Subnet Mask on Win11-Client

Step 4: Check Client Network Configuration

Procedure

On Win11-Client:

- Open Command Prompt.

- Execute:

ipconfig /all

- Locate the Ethernet adapter.

- Check the IPv4 address and subnet mask.

Expected Configuration

IPv4 Address:  172.16.0.50

Subnet Mask:   255.255.255.0

The client's IP may also be another address within the same /24 range.

Observation

The subnet mask is:

255.255.255.0

which corresponds to:

/24

Result

The Win11-Client is configured within the same 172.16.0.0/24 network as the Domain Controller.

Part 5: Practice Subnetting Calculations

Step 5: 192.168.1.0/24

Network Address

192.168.1.0/24

Answer:

192.168.1.0

Broadcast Address

Answer:

192.168.1.255

Usable IP Range

Answer:

192.168.1.1 – 192.168.1.254

Step 6: 10.0.0.0/8

Network Address

10.0.0.0/8

Answer:

10.0.0.0

Broadcast Address

Answer:

10.255.255.255

Usable IP Range

Answer:

10.0.0.1 – 10.255.255.254

Subnetting Practice Summary

| Exercise | Answer |
| --- | --- |
| 192.168.1.0/24 Network Address | 192.168.1.0 |
| 192.168.1.0/24 Broadcast Address | 192.168.1.255 |
| 192.168.1.0/24 Usable IPs | 192.168.1.1 – 192.168.1.254 |
| 10.0.0.0/8 Network Address | 10.0.0.0 |
| 10.0.0.0/8 Broadcast Address | 10.255.255.255 |
| 10.0.0.0/8 Usable IPs | 10.0.0.1 – 10.255.255.254 |

Subnet Mask Reference

| CIDR | Subnet Mask | Total IPs | Usable IPs |
| --- | --- | --- | --- |
| /8 | 255.0.0.0 | 16,777,216 | 16,777,214 |
| /16 | 255.255.0.0 | 65,536 | 65,534 |
| /24 | 255.255.255.0 | 256 | 254 |

Key Takeaways

1. CIDR Prefix

The CIDR prefix indicates how many bits belong to the network portion.

For example:

172.16.0.0/24

means:

Network bits = 24

Host bits    = 8

2. Subnet Mask

For /24:

255.255.255.0

For /16:

255.255.0.0

For /8:

255.0.0.0

3. Total IP Calculation

The basic formula used is:

Total IPs = 2^(Host Bits)

For /24:

Host Bits = 32 - 24

= 8

Total = 2⁸

= 256

4. Usable IP Calculation

For a traditional IPv4 subnet:

Usable IPs = Total IPs - 2

The two reserved addresses are:

Network Address

Broadcast Address

Therefore:

256 - 2 = 254 usable IPs

Skills Demonstrated

- IPv4 Addressing

- CIDR Notation

- Subnetting

- Network Address Calculation

- Broadcast Address Calculation

- Usable IP Range Calculation

- Subnet Mask Identification

- Network Configuration Analysis

- ipconfig /all

- Basic Network Troubleshooting

- Network Address Planning

Conclusion

This lab provided hands-on practice with IPv4 subnetting and network addressing. The network ranges for /24 and /16 were calculated, including their network addresses, broadcast addresses, subnet masks, and usable IP ranges.

The subnet configuration of both the DC-Server and Win11-Client was then verified using ipconfig /all. Finally, subnetting calculations were practiced using 192.168.1.0/24 and 10.0.0.0/8.

The lab strengthened the understanding of CIDR notation, subnet masks, host bits, network addressing, and IP range calculations, which are fundamental networking concepts for cybersecurity and SOC-related work.